CPE301 – SPRING 2020

# Design Assignment 2A

Student Name: Xianjie Cao

Student #: 5004222179

Student Email: caox2@unlv.nevada.edu

Primary Github address: https://github.com/c1029324620/Mocha.git

Directory: Mocha/DesignAssignments/LAB2/DA2A

The goal of the assignment is use GPIO and delays:

1. Design a delay subroutine to generate a waveform on PORTB.3 with 55% DC and 0.75

sec period.

2. Connect a switch to PORTC.3 (active high - turn on the pull up transistor) to poll for an

event to turn on the led at PORTB.2 for 2 sec after the event.

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmel Studio 7: Simulator, debugger and Assembler.

Atmega328PB-Xmini PC

Arduino Multi-function shield: LED, switch

Logic Analyzer

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

Assembly Code for Task 1:

;

; DA2AT1.asm

;

; Created: 2/21/2020 11:50:06 PM

; Author : c1029

;

; design a delay subroutine to generate waveform on PORTB.3 with 55% Duty Cycle and the period is 0.75s.

; Replace with your application code

start:

ldi r16, 0xff	;r16 = 0xff, it will be used to set DDRB as output

out DDRB, r16	;set DDRB as output

again:

sbi PORTB, 3	;set bit 3 in PORTB high.

rjmp delay_high		;jump to delay subroutine, should wait around 0.4s

high_delay_done:

cbi PORTB, 3	;after 0.41 sec delay, make PORTB.3 Low

rjmp delay_low

low_delay_done:

rjmp again		;after 0,33 sec delay, set PORTB.3 High again, then we have generated a waveform.

; 0.4125s delay if it runs at 16 MHz

delay_high:

ldi  r18, 34

ldi  r19, 124

ldi  r20, 86

L1: dec  r20

brne L1

dec  r19

brne L1

dec  r18

brne L1

rjmp high_delay_done

; 0.3375 delay if it runs at 16 MHz

delay_low:

ldi  r18, 28

ldi  r19, 101

ldi  r20, 233

L2: dec  r20

brne L2

dec  r19

brne L2

dec  r18

brne L2

rjmp low_delay_done

done:

rjmp done

C code for Task 1:

/*

* DA2AT1_C.c

*

* Created: 2/22/2020 1:02:46 AM

* Author : c1029

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRB = 0xff; //set PORTB as output

/* Replace with your application code */

while (1)

{

PORTB |= (1 << 3);  //set bit 3 in PORTB to HIGH.

_delay_ms(413);	//wait for 0.4125 sec.

PORTB &= ~(1 << 3);	//set bit 3 in PORTB to LOW.

_delay_ms(338); //wait for delay of 0.3375 sec

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

Assembly Code for Task 2：

;

; DA2AT2.asm

;

; Created: 2/24/2020 8:02:35 PM

; Author : c1029

;

; connect a switch to PORTC.3 to poll for an event to turn on the LED at PORTB.2 for 2 sec.

; Replace with your application code

start:

cbi DDRC, 3		;set PORTC.3 as input

ldi r16, 0x04;set PORTB as output

out DDRB, r16	;set PORTB.2 as output

again:

sbis PINC, 3	;becasue the push button is active high, if the button is push, then will set it low

rjmp delay_2s	;jump to delay and will turn LED on

sbi PORTB, 2	;set bit 2 in PORTB high will set LED off

rjmp again

delay_2s:

cbi PORTB, 2	;this will turn LED on

ldi  r18, 163

ldi  r19, 87

ldi  r20, 3

L1: dec  r20

brne L1

dec  r19

brne L1

dec  r18

brne L1

rjmp again

C code for Task 2:

/*

* DA2AT2_C.c

*

* Created: 2/24/2020 10:07:04 PM

* Author : c1029

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRB |= (1 << 2); //set PORTB.2 as output.

PORTB |= (1 << 2);

DDRC &= (0 << 3); //set PORTC.3 as input.

PORTC |= (1 << 3); //enable pull-up

/* Replace with your application code */

while (1)

{

if(!(PINC & (1 << PINC3)))  // if PINC.3 is pressed.

{

PORTB &= ~(1 << 2);

_delay_ms(2000);

}

else

{

PORTB |= (1 << 2);

}

}

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Delay of 0.415 sec:

Delay of 0.3375 sec:

The Waveform generated on PlusView:

OFF: 478ms – 140ms = 338ms

ON: 891ms – 477ms = 414ms

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

Assembly Demo:

https://youtu.be/VFOdS4pv3yg

C Demo:

https://youtu.be/8GLdofv3Kvw

- GITHUB LINK OF THIS DA

https://github.com/c1029324620/Mocha/tree/master/DesignAssignments/LAB2/DA2A

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Xianjie Cao