CPE301 – SPRING 2020

# Design Assignment 2B

Student Name: Xianjie Cao

Student #: 5004222179

Student Email: caox2@unlv.nevada.edu

Primary Github address: https://github.com/c1029324620/Mocha.git

Directory: Mocha/DesignAssignments/LAB2/DA2B

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmel Studio 7: Simulator, Debugger, and Assembler

Atmega328PB-Xmini PC

Multi-function Shield: Switch, LED

Jump wire

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

Assembly Code for Task 1:

;

; DA2B.asm

;

; Created: 2/27/2020 11:41:16 AM

; Author : c1029

;

; Replace with your application code

.include <m328pbdef.inc>

.org 0	;location for reset

jmp MAIN

.org 0x02	;location for external interrupt 0

jmp EX0_ISR

MAIN:

ldi r20, HIGH(RAMEND)

out SPH, r20

ldi r20, LOW(RAMEND)

out SPL, r20	;initialize stack

cbi DDRC, 3		;set PORTC.3 as input

sbi PORTC, 3	;pull-up activated

ldi r16, 0x04;set PORTB as output

out DDRB, r16	;set PORTB.2 as output

sbi PORTB, 2	;initially, set LED off

ldi r20, 0x02	;make INT0 falling edge triggred

STS EICRA, r20

ldi r20, 1<<INT0	;enable INT0

out EIMSK, r20	;set the bit 0 in the external interrupt mask.

sei

HERE:

jmp HERE

EX0_ISR:

cbi PORTB, 2	;this will turn LED on

delay_2sec:			;delay for 2 sec at 16M Hz

ldi  r18, 163

ldi  r19, 87

ldi  r20, 3

L1: dec  r20

brne L1

dec  r19

brne L1

dec  r18

brne L1

sbi PORTB, 2	;this will turn LED off again

reti

C Code for Task 1:

/*

* DA2B_C.c

*

* Created: 2/27/2020 12:33:59 PM

* Author : c1029

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util//delay.h>

#include <avr/interrupt.h>

int main(void)

{

DDRB |= (1 << 2); //set PORTB.2 as output.

PORTB |= (1 << 2);

DDRC &= (0 << 3); //set PORTC.3 as input.

PORTC |= (1 << 3); //enable pull-up

EICRA = 0x02; //make INT0 falling edge triggered

EIMSK = (1<<INT0);	//enable external interrupt 0

sei();

/* Replace with your application code */

while (1)

{

}

}

ISR(INT0_vect)

{

PORTB &= ~(1 << 2);		//turn LED on

_delay_ms(2000);		//for 2 seconds

PORTB |= (1<< 2);		//then turn it off

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

N/A

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

N/A

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

Assembly: https://youtu.be/33MUaCYDHhg

C: https://youtu.be/6jQyICiEk1w

- GITHUB LINK OF THIS DA

https://github.com/c1029324620/Mocha/tree/master/DesignAssignments/LAB2

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Xianjie Cao