CPE301 – SPRING 2020

# Design Assignment 2C

Student Name: Xianjie Cao

Student #: 5004222179

Student Email: caox2@unlv.nevada.edu

Primary Github address: https://github.com/c1029324620/Mocha.git

Directory: Mocha/DesignAssignments/LAB2/DA2C

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmel Studio 7: Assembler, debugger and simulator.

Atmega328PB-Xmini PC。

Multi-function shield: LED and Pushbutton.

Pins used

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

Task 1 C code:

/*

* DA2CT1.c

*

* Created: 3/4/2020 7:43:16 PM

* Author : c1029

*/

#define F_CPU 16000000UL		//1/16M = 62.5ns for one clock

#include <avr/io.h>

void on_2sec()

{

int cycle = 122;				//2sec/62.5ns/1024/255 = 122 cycles

while(cycle != 0)

{

cycle--;

while((TIFR0 & 0x01) == 0);  //if overflow is set, clear it with reset to 1.

TIFR0 = 1<<TOV0;

}

}

void on_412()

{

int cycle = 25;					//412ms/62.5ns/1024 = 6437.5 clocks to overflow

PORTB |= (1<<3);					//6438/255 = 25 cycles

while(cycle != 0)

{

if(!(PINC &(1<< PINC3)))	//if PINC.3 is pressed.

{

PORTB = 0x08;		//Turn LED on

on_2sec();				//delay for 2 second.

PORTB |=(1<<2);			//Turn LED off.

}

cycle--;

while((TIFR0 & 0x01) == 0);	//if overflow is set, clear it with reset to 1.

TIFR0 |= 1<<TOV0;

}

}

void off_338()

{

int cycle = 20;			//338ms/62.5ns/1024 = 5281.25 clocks to overflow

PORTB &=~(1<<3);			//5281.25/255 = 20 cycles

while(cycle != 0)

{

if(!(PINC &(1<< PINC3)))	//if PINC.3 is pressed.

{

PORTB = 0x08;		//Turn LED on.

on_2sec();				//delay for 2 second.

PORTB |=(1<<2);			//Turn LED off.

}

cycle--;

while((TIFR0 & 0x01) == 0);

TIFR0 = 1<<TOV0;		//clear overflow flag

}

}

int main(void)

{

DDRC &= ~(1<<3);	//set PORTC.3 as input

DDRB = 0x0C;	//set PORTB.3 and PORTB.2 as output

PORTC |= (1<<3); //active pull-up resistor for pushbutton

PORTB |= (1<<2);	//pull-up resistor for LED

/* Replace with your application code */

while (1)

{

TCCR0A = 0x00;		//normal mode operation

TCCR0B = 0x05;		//prescaler of 1024

TCNT0 = 0X00;

on_412();			//hold PORTB.3 HIGH for 412ms

off_338();			//hold PORTB.3 LOW for 338ms

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

Task 2 C code:

/*

* DA2CT2.c

*

* Created: 3/4/2020 9:58:21 PM

* Author : c1029

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

volatile uint8_t cycle_cnt;		//global variable modified by ISR.

int main(void)

{

DDRC &= ~(1<<3);	//set PORTC.3 as input

DDRB = 0x0C;	//set PORTB.3 and PORTB.2 as output

PORTC |= (1<<3);	//active pull-up resistor for pushbutton

PORTB |= (1<<2) | (1 << 3);	//pull-up resistor for LED and PORTB.3

TCNT0 = 0x00;

TCCR0A = 0x00;		//normal mode operation

TCCR0B = 0x05;		//prescaler of 1024

TIMSK0 = (1<<TOIE0);	//enable timer0 overflow interrupt

sei();

/* Replace with your application code */

while (1)

{

if(!(PINC & (1<<PINC3)))

{

cycle_cnt = 0;

PORTB = 0x08;

while(cycle_cnt < 123);

PORTB |=(1<<2);

cycle_cnt = 0;

}

if((cycle_cnt < 45) && (cycle_cnt >= 20))

{

PORTB |= (1 << 3);  //set PORTB.3 HIGH

}

if(cycle_cnt > 45)

{

PORTB &= ~(1<<3);

cycle_cnt = 0;		//set PORTB.3 LOW

}

}

}

ISR(TIMER0_OVF_vect)

{

cycle_cnt++;

}

Task 3 C code:

/*

* DA2CT3.c

*

* Created: 3/4/2020 11:05:15 PM

* Author : c1029

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

volatile uint8_t cycle_cnt;		//global variable modified by ISR.

int main(void)

{

DDRC &= ~(1<<3);	//set PORTC.3 as input

DDRB = 0x0C;	//set PORTB.3 and PORTB.2 as output

PORTC |= (1<<3);	//active pull-up resistor for pushbutton

PORTB |= (1<<2) | (1 << 3);	//pull-up resistor for LED and PORTB.3

TCNT0 = 0x00;

TCCR0A = (1<<WGM01);		//CTC mode operation

TCCR0B = 0x05;		//prescaler of 1024

OCR0A = 0x80;		//overflow happens when TCNT exceed 0x80 or 128

TIMSK0 = (1<<OCIE0A);	//enable timer0 compare interrupt

sei();

/* Replace with your application code */

while (1)

{

if(!(PINC & (1<<PINC3)))

{

cycle_cnt = 0;

PORTB = 0x08;

while(cycle_cnt < 245);

PORTB |=(1<<2);

cycle_cnt = 0;

}

if((cycle_cnt < 90) && (cycle_cnt >= 40))

{

PORTB |= (1 << 3);  //set PORTB.3 HIGH

}

if(cycle_cnt > 90)

{

PORTB &= ~(1<<3);

cycle_cnt = 0;		//set PORTB.3 LOW

}

}

}

ISR(TIMER0_COMPA_vect)

{

cycle_cnt++;

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

On for 0.42 sec:

Off for 0.33 sec:

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

Task 1: https://youtu.be/azjUurHv-o0

Task 2: https://youtu.be/5ENykd-rViE

Task 3: https://youtu.be/EeoZShHN83c

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Xianjie Cao