CPE301 – SPRING 2020

# Design Assignment 4A

Student Name: Xianjie Cao

Student #: 5004222179

Student Email: caox2@unlv.nevada.edu

Primary Github address: https://github.com/c1029324620/Mocha.git

Directory: Mocha/DesignAssignments/LAB4/DA4A

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmel Studio 7: Debugger, simulator and assembler

Atmega328pb Xmini PC

DC Motor

Breadboard

Jump wires

TB6612FNG

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* DA4A.c

*

* Created: 3/30/2020 12:54:04 PM

* Author : c1029

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

#include <avr/interrupt.h>

unsigned int adc_value;

void read_adc()

{

ADCSRA |= (1 << ADSC);		//enable start conversion

while(ADCSRA & (1<< ADSC));	//wait

adc_value = ADC;	// potentiometer value stored in adc_value

}

void init_adc()

{

ADMUX = (0<<REFS1) |	//AVref

(1<<REFS0) |

(1<<ADLAR) |	//Left justified

(0<<MUX2)  |	//ADC0 channel 0

(0<<MUX1)  |

(0<<MUX0);

ADCSRA = (1<<ADEN) |	//Enable ADC

(0<<ADSC) |

(0<<ADATE)|

(0<<ADIF) |

(0<<ADIE) |

(1<<ADPS2)|	//prescaler of 128

(1<<ADPS1)|

(1<<ADPS0);

}

int main(void)

{

/* Replace with your application code */

init_adc();

DDRC |= (0<<DDRC1);	//pinc.1 as input

PORTC |= (1 << 1); //pull=up resistor

PCICR |= (1<<PCIE1); //pin change interrupt

PCMSK1 |= (1<<PCINT9);

sei();

DDRB |= (1<< DDRB1);	//Port B.1 as output

ICR1 = 0xffff;

TCCR1A = (1<<COM1A1) | (1<<COM1B1) | (1<<WGM11);		//non-inverting

TCCR1B = (1<< WGM12) | (1<<WGM13);	//Fast PWM using ICR1 as TOP

TCCR1B = (1<< CS10);	// no prescaler.

while(1)

{

read_adc();

if((adc_value >= 62258) && (adc_value < 65535))	//95% of PWM value

{

OCR1A = 62258;

_delay_ms(20);

}

else if((adc_value < 62257) && (adc_value >= 3277))

{

OCR1A = adc_value;

_delay_ms(20);

}

else

OCR1A = 0x00;		//minimum speed at less than 5% of PWM value.

}

return 0;

}

ISR(PCINT1_vect)

{

DDRC ^=(1<<2);

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

N/A

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

N/A

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/EMT-5ujx1WQ

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Xianjie Cao