CPE301 – SPRING 2020

# Design Assignment 5

Student Name: Xianjie Cao

Student #: 5004222179

Student Email: caox2@unlv.nevada.edu

Primary Github address: https://github.com/c1029324620/Mocha.git

Directory: Mocha/DesignAssignments/Lab5

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmel Studio 7: Debugger, simulator, and assembler.

Atmega328pb X-mini

Jumper wires

Multifunctional Shield

LM35

DS18b20

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

Task 1:

#include <avr/io.h>

#include <stdio.h>

#ifndef F_CPU

#define F_CPU 16000000UL

#endif

#ifndef BAUD

#define BAUD 9600

#endif

#include <stdlib.h>

#include <util/delay.h>

#include <util/setbaud.h>

#define LATCH 4 /* PD4 RCK */

#define CLOCK 7 /* PD7 SRCK */

#define DATA 0  /* PB0 SER IN */

#define LSBFIRST 0

#define MSBFIRST 1

unsigned int adc_value;

//initialized adc module

void adc_int(void)

{

//setup and enable ADC

ADMUX = (0 << REFS1) | //reference selection bits

(1 << REFS0) |	//AVcc-external cap at AREF

(0 << ADLAR) | //ADC left adjust result

(1 << MUX2)  |	//analog channel selection bits

(0 << MUX1)  |	// ADC4(PC4)

(0 << MUX0);

ADCSRA = (1 << ADEN) |	// ADC enable

(0 << ADSC) | // ADC start conversion

(0 << ADATE) | // ADC auto trigger enable

(0 << ADIF) | // ADC interrupt flag

(0 << ADIE) | // ADC interrupt Enable

(1 << ADPS2) | // ADC prescaler select bits

(0 << ADPS1) |

(1 << ADPS0);

}

void adc_read(void)

{

ADCSRA |= (1 << ADSC);		//enable start conversion

while(ADCSRA & (1<< ADSC));	//wait

adc_value = ADC;	// LM35 value stored in temp.

}

/* Segment byte maps for numbers 0 to 9 */

const uint8_t SEGMENT_MAP[] = {0xC0, 0xF9, 0xA4, 0xB0, 0x99,

0x92, 0x82, 0xF8, 0X80, 0X90};

/* Byte maps to select digit 1 to 4 */

const uint8_t SEGMENT_SELECT[] = {0xF1, 0xF2, 0xF4, 0xF8};

void shift_out_init(void) {

DDRB |= (1 << DATA);

DDRD |= (1 << CLOCK) | (1 << LATCH);

}

void uart_init(void) {

UBRR0H = UBRRH_VALUE;

UBRR0L = UBRRL_VALUE;

#if USE_2X

UCSR0A |= _BV(U2X0);

#else

UCSR0A &= ~(_BV(U2X0));

#endif

UCSR0C = _BV(UCSZ01) | _BV(UCSZ00); /* 8-bit data */

UCSR0B = _BV(RXEN0) | _BV(TXEN0);   /* Enable RX and TX */

}

int uart_putchar(char c, FILE *stream) {

if (c == '\n') {

uart_putchar('\r', stream);

}

loop_until_bit_is_set(UCSR0A, UDRE0);

UDR0 = c;

return 0;

}

int uart_getchar(FILE *stream) {

loop_until_bit_is_set(UCSR0A, RXC0);

return UDR0;

}

FILE uart_output = FDEV_SETUP_STREAM(uart_putchar, NULL, _FDEV_SETUP_WRITE);

FILE uart_input = FDEV_SETUP_STREAM(NULL, uart_getchar, _FDEV_SETUP_READ);

void shift_out(uint8_t indata) {

for (uint8_t i = 0; i < 8; i++) {

/* Write bit to data port. */

if (0 == (indata & _BV(7 - i))) {

// digital_write(SHIFT_OUT_DATA, LOW);

PORTB &= (0 << DATA);

} else {

// digital_write(SHIFT_OUT_DATA, HIGH);

PORTB |= (1 << DATA);

}

/* Pulse clock to write next bit. */

PORTD |= (1 << CLOCK);

PORTD &= (0 << CLOCK);

}

}

void shiftOut(uint8_t dataPin, uint8_t clockPin, uint8_t bitOrder,

uint8_t val) {

uint8_t i;

for (i = 0; i < 8; i++) {

if (bitOrder == LSBFIRST)

dataPin |= !!(val & (1 << i));

else

dataPin |= !!(val & (1 << (7 - i)));

PORTD |= (1 << CLOCK);

PORTD &= (0 << CLOCK);

}

}

void shift_out_latch(void) {

PORTD &= (0 << LATCH);

PORTD |= (1 << LATCH);

}

int main(void) {

shift_out_init();

uart_init();

stdout = &uart_output;

stdin = &uart_input;

char binary[17];

adc_int();

while (1) {

adc_read();

int first_digi = adc_value % 10;		//get the first digit

int second_digi = adc_value / 10;		//get the ten digit

int third_digi = adc_value / 100;		//get the hundreds digit

PORTD &= (0<< LATCH);

shift_out(SEGMENT_MAP[third_digi]);

shift_out(0xF2);

PORTD |= (1<< LATCH);

PORTD &= (0<< LATCH);

shift_out(SEGMENT_MAP[second_digi]);

shift_out(0xF4);

PORTD |= (1<< LATCH);

PORTD &= (0<< LATCH);

shift_out(SEGMENT_MAP[first_digi]);

shift_out(0xF8);

PORTD |= (1<< LATCH);

}

return 0;

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

Task 2:

/*

* SPI_ShiftRegister.c

*

* Created: 4/2/2020 12:55:58 AM

* Author : VenkatesanMuthukumar

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#define SHIFT_REGISTER DDRB

#define SHIFT_PORT PORTB

#define DATA (1<<PB3)     //MOSI (SI)

#define LATCH (1<<PB2)        //SS   (RCK)

#define CLOCK (1<<PB5)        //SCK  (SCK)

/* Note the mapping for Multifunctional shield

Name		-  Board       -    Shield

DATA/MOSI		PB3	(11)		PB0 (8)

LATCH/SS		PB2	(10)		PD4 (4)

CLOCK/SCK		PB5	(13)		PD7 (7)

-/MISO			PB4	(12)		PB1	(9)

*/

uint8_t adc_value;

/* Segment byte maps for numbers 0 to 9 */

const uint8_t SEGMENT_MAP[] = {0xC0, 0xF9, 0xA4, 0xB0, 0x99,

0x92, 0x82, 0xF8, 0X80, 0X90};

/* Byte maps to select digit 1 to 4 */

const uint8_t SEGMENT_SELECT[] = {0xF1, 0xF2, 0xF4, 0xF8};

//initialized adc module

void adc_int(void)

{

//setup and enable ADC

ADMUX = (0 << REFS1) | //reference selection bits

(1 << REFS0) |	//AVcc-external cap at AREF

(0 << ADLAR) | //ADC left adjust result

(1 << MUX2)  |	//analog channel selection bits

(0 << MUX1)  |	// ADC4(PC4)

(0 << MUX0);

ADCSRA = (1 << ADEN) |	// ADC enable

(0 << ADSC) | // ADC start conversion

(0 << ADATE) | // ADC auto trigger enable

(0 << ADIF) | // ADC interrupt flag

(0 << ADIE) | // ADC interrupt Enable

(1 << ADPS2) | // ADC prescaler select bits

(0 << ADPS1) |

(1 << ADPS0);

}

void adc_read(void)

{

ADCSRA |= (1 << ADSC);		//enable start conversion

while(ADCSRA & (1<< ADSC));	//wait

adc_value = ADC;	// LM35 value stored in temp.

}

int main(void)

{

adc_int();		//initialize the adc module

while(1)

{

adc_read();

SHIFT_REGISTER |= (DATA | LATCH | CLOCK); //Set control pins as outputs

SHIFT_PORT &= ~(DATA | LATCH | CLOCK);        //Set control pins low

SPCR0 = (1<<SPE) | (1<<MSTR);  //Start SPI as Master

SHIFT_PORT &= ~LATCH;

//Shift in some data

SPDR0 = SEGMENT_MAP[adc_value % 10];        //This should light alternating LEDs

//Wait for SPI process to finish

while(!(SPSR0 & (1<<SPIF)));

//Shift in some more data since I have two shift registers hooked up

SPDR0 = 0xF8;        //This should light alternating LEDs

//Wait for SPI process to finish

while(!(SPSR0 & (1<<SPIF)));

//Toggle latch to copy data to the storage register

SHIFT_PORT |= LATCH;

SHIFT_PORT &= ~LATCH;

SPDR0 = SEGMENT_MAP[adc_value /10];        //This should light alternating LEDs

//Wait for SPI process to finish

while(!(SPSR0 & (1<<SPIF)));

//Shift in some more data since I have two shift registers hooked up

SPDR0 = 0xF4;        //This should light alternating LEDs

//Wait for SPI process to finish

while(!(SPSR0 & (1<<SPIF)));

//Toggle latch to copy data to the storage register

SHIFT_PORT |= LATCH;

SHIFT_PORT &= ~LATCH;

SPDR0 = SEGMENT_MAP[adc_value /100];        //This should light alternating LEDs

//Wait for SPI process to finish

while(!(SPSR0 & (1<<SPIF)));

//Shift in some more data since I have two shift registers hooked up

SPDR0 = 0xF2;        //This should light alternating LEDs

//Wait for SPI process to finish

while(!(SPSR0 & (1<<SPIF)));

//Toggle latch to copy data to the storage register

SHIFT_PORT |= LATCH;

SHIFT_PORT &= ~LATCH;

//Loop forever

}

Task 3:

//Define MCU CPU Freq for the time delay functions

#ifndef F_CPU

#define F_CPU 16000000UL

#endif

#include <stdlib.h>

#include <string.h>

#include <avr/interrupt.h>

#include <util/delay.h>

#include "ds18b20.h"

#include "uart.h"

int main(void) {

char printbuff[100];

double d = 0;

//init uart

USART_Init( 9600);

//init interrupt

sei();

while(1) {

d = ds18b20_gettemp();

dtostrf(d, 10, 3, printbuff);

USART_SendString("Temperature: ");

USART_SendString(printbuff);

USART_SendString("\r\n");

_delay_ms(500);

}

return 0;

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Task 1:

Task 2:

Task 3:

- VIDEO LINKS OF EACH DEMO

Task 1: https://youtu.be/KI4gFGT-7sg

Task 2: https://youtu.be/A_D5ACGhk4k

Task 3: https://youtu.be/2aFyYCPRc7M

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Xianjie Cao