CPE301 – SPRING 2020

# Design Assignment 6

Student Name: Xianjie Cao

Student #: 5004222179

Student Email: caox2@unlv.nevada.edu

Primary Github address: https://github.com/c1029324620/Mocha.git

Directory: Mocha/DesignAssignments/Lab6

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmel Studio 7: Assembler, debugger and simulator

Atmega328pb-Xmini

Jumper wires

Breadboard

MPU6050

Multifunctional Shield

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

#define F_CPU 16000000UL									/* Define CPU clock Frequency e.g. here its 8MHz */

#include <avr/io.h>										/* Include AVR std. library file */

#include <util/delay.h>									/* Include delay header file */

#include <inttypes.h>									/* Include integer type header file */

#include <stdlib.h>										/* Include standard library file */

#include <stdio.h>

#include <math.h>									/* Include standard library file */

#include "MPU6050_def.h"							/* Include MPU6050 register define file */

#include "i2c_master.h"							/* Include I2C Master header file */

#include "uart.h"							/* Include USART header file */

float Acc_x,Acc_y,Acc_z,Temperature,Gyro_x,Gyro_y,Gyro_z;

void MPU6050_Init()										/* Gyro initialization function */

{

_delay_ms(150);										/* Power up time >100ms */

I2C_Start_Wait(0xD0);								/* Start with device write address */

I2C_Write(SMPLRT_DIV);								/* Write to sample rate register */

I2C_Write(0x07);									/* 1KHz sample rate */

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(PWR_MGMT_1);								/* Write to power management register */

I2C_Write(0x01);									/* X axis gyroscope reference frequency */

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(CONFIG);									/* Write to Configuration register */

I2C_Write(0x00);									/* Fs = 8KHz */

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(GYRO_CONFIG);								/* Write to Gyro configuration register */

I2C_Write(0x18);									/* Full scale range +/- 2000 degree/C */

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(INT_ENABLE);								/* Write to interrupt enable register */

I2C_Write(0x01);

I2C_Stop();

}

void MPU_Start_Loc()

{

I2C_Start_Wait(0xD0);								/* I2C start with device write address */

I2C_Write(ACCEL_XOUT_H);							/* Write start location address from where to read */

I2C_Repeated_Start(0xD1);							/* I2C start with device read address */

}

void Read_RawValue()

{

MPU_Start_Loc();									/* Read Gyro values */

Acc_x = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Acc_y = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Acc_z = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Temperature = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Gyro_x = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Gyro_y = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Gyro_z = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Nack());

I2C_Stop();

}

int main()

{

//Variable Declarations

char buffer[20];

char float_val[10];

float X_a,Y_a,Z_a;			//Acceleration Values X-Y-Z

float X_g=0,Y_g=0,Z_g=0;		//Gyro Values X-Y-Z

I2C_Init();						//Initialize I2C

MPU6050_Init();					//Initialize MPU6050

USART_Init(9600);				//Initialize UART

while(1)

{

Read_RawValue();			//Call Read_RawValue Function

X_a = Acc_x/16384.0;		//Acceleration-X

Y_a = Acc_y/16384.0;		//Acceleration-Y

Z_a = Acc_z/16384.0;		//Acceleration-Z

X_g = Gyro_x/16.4;			//Gyro-X

Y_g = Gyro_y/16.4;			//Gyro-Y

Z_g = Gyro_z/16.4;			//Gyro-Z

//Read/Print Acceleration-X

dtostrf( X_a, 3, 2, float_val );

sprintf(buffer," Ax = %s g\t",float_val);

USART_SendString(buffer);

//Read/Print Acceleration-Y

dtostrf( Y_a, 3, 2, float_val );

sprintf(buffer," Ay = %s g\t",float_val);

USART_SendString(buffer);

//Read/Print Acceleration-Z

dtostrf( Z_a, 3, 2, float_val );

sprintf(buffer," Az = %s g\t",float_val);

USART_SendString(buffer);

//Read/Print Gyro-X

dtostrf( X_g, 3, 2, float_val );

sprintf(buffer," Gx = %s%c/s\t",float_val,0xF8);

USART_SendString(buffer);

//Read/Print Gyro-Y

dtostrf( Y_g, 3, 2, float_val );

sprintf(buffer," Gy = %s%c/s\t",float_val,0xF8);

USART_SendString(buffer);

//Read/Print Gyro-Z

dtostrf( Z_g, 3, 2, float_val);

sprintf(buffer," Gz = %s%c/s\r\n",float_val,0xF8);

USART_SendString(buffer);

_delay_ms(1000);

}

}//End Main

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

#define F_CPU 16000000UL									/* Define CPU clock Frequency e.g. here its 8MHz */

#include <avr/io.h>										/* Include AVR std. library file */

#include <util/delay.h>									/* Include delay header file */

#include <inttypes.h>									/* Include integer type header file */

#include <stdlib.h>										/* Include standard library file */

#include <stdio.h>

#include <math.h>									/* Include standard library file */

#include "MPU6050_def.h"							/* Include MPU6050 register define file */

#include "i2c_master.h"							/* Include I2C Master header file */

#include "uart.h"							/* Include USART header file */

float Acc_x,Acc_y,Acc_z,Temperature,Gyro_x,Gyro_y,Gyro_z, pitch, roll;

#define ACCELEROMETER_SENSITIVITY 16384.0

#define GYROSCOPE_SENSITIVITY 16.4

#define dt 0.01   // 10 ms sample rate!

void MPU6050_Init()										/* Gyro initialization function */

{

_delay_ms(150);										/* Power up time >100ms */

I2C_Start_Wait(0xD0);								/* Start with device write address */

I2C_Write(SMPLRT_DIV);								/* Write to sample rate register */

I2C_Write(0x07);									/* 1KHz sample rate */

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(PWR_MGMT_1);								/* Write to power management register */

I2C_Write(0x01);									/* X axis gyroscope reference frequency */

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(CONFIG);									/* Write to Configuration register */

I2C_Write(0x00);									/* Fs = 8KHz */

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(GYRO_CONFIG);								/* Write to Gyro configuration register */

I2C_Write(0x18);									/* Full scale range +/- 2000 degree/C */

I2C_Stop();

I2C_Start_Wait(0xD0);

I2C_Write(INT_ENABLE);								/* Write to interrupt enable register */

I2C_Write(0x01);

I2C_Stop();

}

void MPU_Start_Loc()

{

I2C_Start_Wait(0xD0);								/* I2C start with device write address */

I2C_Write(ACCEL_XOUT_H);							/* Write start location address from where to read */

I2C_Repeated_Start(0xD1);							/* I2C start with device read address */

}

void Read_RawValue()

{

MPU_Start_Loc();									/* Read Gyro values */

Acc_x = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Acc_y = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Acc_z = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Temperature = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Gyro_x = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Gyro_y = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

Gyro_z = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Nack());

I2C_Stop();

}

void getAngle(int Ax, int Ay, int Az)

{

double x = Ax;

double y = Ay;

double z = Az;

pitch = atan(x / sqrt((y*y) + (z*z)));		//pitch calculation

roll = atan(y/ sqrt((x*x) + (z*z)));		//roll calculation

//converting radians into degrees

pitch = pitch * (180.0 / 3.14);

roll = roll * (180.0 / 3.14);

}

void ComplementaryFilter()

{

float pitchAcc, rollAcc;

// Integrate the gyroscope data -> int(angularSpeed) = angle

pitch += ((float)Gyro_x / GYROSCOPE_SENSITIVITY) * dt;

// Angle around the X-axis

roll -= ((float)Gyro_y / GYROSCOPE_SENSITIVITY) * dt;

// Angle around the Y-axis

// Compensate for drift with accelerometer data if !bullshit

// Sensitivity = -2 to 2 G at 16Bit -> 2G = 32768 && 0.5G = 8192

int forceMagnitudeApprox = abs(Acc_x) + abs(Acc_y) + abs(Acc_z);

if (forceMagnitudeApprox > 8192 && forceMagnitudeApprox < 32768)

{

// Turning around the X axis results in a vector on the Y-axis

pitchAcc = atan2f((float)Acc_y, (float)Acc_z) * 180 / M_PI;

pitch = pitch * 0.98 + pitchAcc * 0.02;

// Turning around the Y axis results in a vector on the X-axis

rollAcc = atan2f((float)Acc_x, (float)Acc_z) * 180 / M_PI;

roll = roll * 0.98 + rollAcc * 0.02;

}

}

int main()

{

//Variable Declarations

char buffer[20];

char float_val[10];

//float X_a,Y_a,Z_a;			//Acceleration Values X-Y-Z

//float X_g=0,Y_g=0,Z_g=0;		//Gyro Values X-Y-Z

I2C_Init();						//Initialize I2C

MPU6050_Init();					//Initialize MPU6050

USART_Init(9600);				//Initialize UART

//10ms sample

TCCR1B = (1<< CS10);		//prescaler of 8

TCNT1 = 0;

while(1)

{

if(TCNT1 == 20000)

{

Read_RawValue();			//Call Read_RawValue Function

/*	X_a = Acc_x/16384.0;		//Acceleration-X

Y_a = Acc_y/16384.0;		//Acceleration-Y

Z_a = Acc_z/16384.0;		//Acceleration-Z

X_g = Gyro_x/16.4;			//Gyro-X

Y_g = Gyro_y/16.4;			//Gyro-Y

Z_g = Gyro_z/16.4;			//Gyro-Z*/

getAngle(Acc_x, Acc_y, Acc_z);

ComplementaryFilter();

//Read/Print pitch

dtostrf(pitch, 3, 2, float_val );

sprintf(buffer," Pitch = %s\t",float_val);

USART_SendString(buffer);

//Read/Print Gyro-Z

dtostrf(roll, 3, 2, float_val);

sprintf(buffer," Roll = %s\n",float_val);

USART_SendString(buffer);

TCNT1 = 0;	//reset timer counter

}

}

}//End Main

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Task 1:

Task 2:

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

Task 1: https://youtu.be/Uiu1zLGxVh8

Task 2: https://youtu.be/01EThdNeow0

- GITHUB LINK OF THIS DA

https://github.com/c1029324620/Mocha/tree/master/DesignAssignments/LAB6/DA6

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Xianjie Cao