CPE301 – SPRING 2020

# MIDTERM 1

Student Name: Xianjie Cao

Student #: 5004222179

Student Email: caox2@unlv.nevada.edu

Primary Github address: https://github.com/c1029324620/Mocha.git

Directory: Mocha/Midterm

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmel Studio 7: simulator, debugger, assembler, terminal.

LM 35

Atmega328PB-Xmini PC

LED

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* Midterm.c

*

* Created: 3/20/2020 10:57:17 AM

* Author : c1029

*/

#define F_CPU 16000000UL

#define BAUD 9600

#include <avr/io.h>

#include <util/setbaud.h>

#include <stdlib.h>

#include <stdio.h>

#include <util\delay.h>

#include <time.h>

#include <avr/interrupt.h>

volatile uint8_t temp;

void USART_init(void)

{

UBRR0H = UBRRH_VALUE;

UBRR0L = UBRRL_VALUE;

UCSR0C = _BV(UCSZ01) | _BV(UCSZ00);		//8-bit data

UCSR0B = _BV(RXEN0) | _BV(TXEN0);		//enable receiver and transmitter

}

void adc_int(void)

{

//setup and enable ADC

ADMUX = (0 << REFS1) | //reference selection bits

(1 << REFS0) |	//AVcc-external cap at AREF

(0 << ADLAR) | //ADC left adjust result

(1 << MUX2)  |	//analog chanel selection bits

(0 << MUX1)  |	// ADC4(PC4)

(0 << MUX0);

ADCSRA = (1 << ADEN) |	// ADC enable

(0 << ADSC) | // ADC start conversion

(0 << ADATE) | // ADC auto trigger enable

(0 << ADIF) | // ADC interrupt flag

(0 << ADIE) | // ADC interrupt Enable

(1 << ADPS2) | // ADC prescaler select bits

(0 << ADPS1) |

(1 << ADPS0);

}

void adc_read(void)

{

ADCSRA |= (1 << ADSC);		//enable start conversion

while(ADCSRA & (1<< ADSC));	//wait

temp = ADC;	// LM35 value stored in temp.

}

//send some data to the serial port

void USART_tx_string(char* data)

{

while((*data !='\0'))

{

while(!(UCSR0A & (1<<UDRE0)));

UDR0 = *data;

data++;

}

}

char USART_Receiver()

{

//wait for data to be received

while(!(UCSR0A & (1<< RXC0)));

//get and return received data from buffer

return UDR0;

}

void display_help()

{

USART_tx_string("***'t' display temperature in Celsius.\n");

USART_tx_string("***'T' display temperature in Farenthide.\n");

USART_tx_string("***'o' turns ON LED at PB5.\n***'O' turns OFF the LED at PB5.\n");

USART_tx_string("***'s' sends a string to the terminal.\n");

USART_tx_string("***'i' sends an integer to the terminal.\n");

USART_tx_string("Press 'h' key to display again the help table.\n");

}

void display_tem_c()

{

adc_read();

char array1[20];

int celsius = (temp - 32) * 5 / 9;		//convert to celsius

sprintf(array1,"%i", celsius );			//convert int to string

USART_tx_string("Temperature: ");		//print

USART_tx_string(array1);

USART_tx_string(" C\n");

}

void display_tem_t()

{

adc_read();

char array1[20];

sprintf(array1,"%i", temp);			//convert int to string

USART_tx_string("Temperature: ");	//print

USART_tx_string(array1);

USART_tx_string(" F\n");

}

void turn_on()

{

PORTB &= ~(1 << 5);  //Turn LED on

}

void turn_off()

{

PORTB |= (1 << 5);	//Turn LED off

}

void send_str()

{

USART_tx_string("Hello World!\n");		//send "Hello World" to terminal

}

void my_delay_ms(int n) {

while(n--) {

_delay_ms(1);

}

}

void send_int()

{

int delay = (rand() % 3 + 1) * 1000;		//generate random numbers of 1000, 2000, 3000 for delay in ms

char array1[20];

sprintf(array1, "%i", delay);

USART_tx_string(array1);

USART_tx_string("\n");

turn_on();

my_delay_ms(delay);

turn_off();

my_delay_ms(delay);

}

int main(void)

{

srand(time(NULL));

/* Replace with your application code */

USART_init();		//initialized USART module

adc_int();			//initialized ADC module

DDRB |= (1 << 5); //set PORTB.5 as output

PORTB |= (1 << 5);	//pull up;

display_help();

while (1)

{

switch(USART_Receiver())

{

case 'h':

display_help();			//display help table

break;

case 't':

display_tem_c();		//display temperature in C

break;

case 'T':

display_tem_t();		//display temperature in F

break;

case 'o':

turn_on();				//turn on LED

break;

case 'O':

turn_off();				//turn off LED

break;

case 's':

send_str();				//send string

break;

case 'i':

send_int();				//send integer, and blink LED

break;

default:

break;

}

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

N/A

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/_FAkqU4I3m8

- GITHUB LINK OF THIS DA

https://github.com/c1029324620/Mocha/tree/master/Midterm

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Xianjie Cao